Polycarbon wind speed transmitter

Analog quantity type

PR-3000-FSJT-*

VER 2.0

Table of contents

Chapter 1 product introduction

1.1 product overview

1.2 functional characteristics

1.3 main parameters

1.4 product selection

Chapter 2 hardware connectivity

2.1 equipment inspection before installation

2.2

2.2.1 sensor wiring

2.3 installation mode

2.4 points to note

Chapter 3 wiring instructions

Chapter 4 the meaning of analog parameter

Chapter 1 Product Introduction

1.1 product overview

PR-3000-FSJT-* wind speed transmitter, small and portable, easy to carry and assemble, three cups design concept can effectively obtain wind speed information, the shell is made of polycarbonate composite material, with good corrosion resistance, corrosion resistance and other characteristics, it can ensure that the transmitter is used for a long time without the phenomenon of rust-cut, at the same time, it can cooperate with the internal smooth bearing system to ensure the accuracy of information collection, and output the data with the traditional analog signals (4-20mA, 0-10V, 0-5V) . It is widely used in greenhouse, environmental protection, weather stations, ships, docks, aquaculture and other environmental wind speed measurement.

1.2 functional characteristics

Range: 0-30m/s (customizable 0-70m/s) , Resolution 0.1 m/s

Anti-electromagnetic interference processing

Bottom outlet mode, completely eliminate the aging problem of aviation plug rubber pad, long-term use is still waterproof

Adopt high-performance imported bearings, small rotational resistance, accurate measurement

Polycarbonate shell, mechanical strength, high hardness, corrosion resistance, non-rust can be used outdoors for a long time

The structure and weight of the equipment are carefully designed and distributed, and the moment of inertia is small and the response is sensitive

The utility model can be applied to four-wire system and three-wire system connection at the same time.

1.3 main parameters

| DC power supply (default) | 10-30 V DC |  |
| --- | --- | --- |
| Maximum power consumption | Current output | 1.2 W |
|  | Voltage output | 1.2 W |
| Resolution | 0.1 m/s |  |
| Precision | ± (0.2 + 0.03 V) m/S V indicates wind speed |  |
| Working temperature of transmitter circuit | -20 ° C ~ + 60 ° C, 0% RH ~ 80% Rh |  |
| Measuring Range | Default 0-30m/s (customizable to 0-70m/s) |  |
| Start speed | ≤0.2 m/s |  |
| Dynamic response time | ≤1s |  |
| Output signal | Current output | 4 ~ 20 ma |
|  | Voltage output | 0 ~ 5V/0 ~ 10V |
| Load capacity | Voltage output | Output resistance ≤250ω |
|  | Current output | ≤600 ω |

For long-term use, please keep the ambient wind speed below 30m/s

Product size:

1.4 product selection

| PR - |  |  |  | Company code name |
| --- | --- | --- | --- | --- |
|  | 3000 - |  |  | Shell code |
|  |  | FSJT - |  | Wind speed transmitter |
|  |  |  | I20 | 4-20ma current output |
|  |  |  | V05 | 0-5v voltage output |
|  |  |  | V10 | 0-10V voltage output |

Chapter 2 hardware connectivity

2.1 equipment inspection before installation

Equipment list:

1 transmitter equipment

‖ install 4 screws

The Certificate of Conformity and Warranty Card

2.2

Wide voltage 10 ~ 30V DC power input. For 0-10V output devices can only use 24V power supply.

2.2.1 sensor wiring

|  | Line color | Description |
| --- | --- | --- |
| Power supply | Brown | The power supply is positive |
|  | Black | The power supply is negative |
| Output | Blue | The wind speed signal is positive |
|  | Green | The wind speed signal is negative |

2.3 installation mode

The lower pipe of the wind speed sensor is firmly fixed on the flange plate by means of flange installation and threaded flange connection. The chassis ø80mm is provided with four mounting holes in the circumference of the Ø68MM which are all ø4.5 mm, use Bolts to fasten it on the bracket to keep the whole set of instruments at the best level, ensure the accuracy of wind speed data, flange connection is convenient to use, can withstand greater pressure.

2.4 points to note

1. Users are not allowed to disassemble, but also can not touch the sensor core, so as not to cause damage to the product.

2. As far as possible away from high-power interference equipment, so as not to cause inaccurate measurement, such as inverter, motor, etc. .

3. To prevent chemical reagents, oil, dust and other direct damage to the sensor, do not dew, limit temperature environment for long-term use, prevent cold and heat shock

Chapter 3 wiring instructions

Analog volume sensor wiring is simple, only the wire and the device can be designated port connection. Equipment Standard is with 2 independent analog output. At the same time to adapt to the three-wire system and four-wire system.

Schematic diagram of four-wire connection method

Schematic diagram of three-wire connection method

Chapter 4 the meaning of analog parameter

4.1 current-mode output signal conversion calculation

Range 0 ~ 30m/s, 4 ~ 20mA output, when the output signal 12mA, calculate the current wind speed. The span of wind speed range is 30m/s, expressed by 16mA current signal, 30M/s/16mA = 1.875 m/s/MA, that is, the change of current is 1MA, the change of wind speed is 1.875 m/s. Then you can calculate the measurements: 12ma-4ma = 8ma. 8MA * 1.875 m/s/MA = 15m/s, then the current wind speed is 15m/s.

4.2 voltage type output signal conversion calculation

The range is 0 ~ 30m/s, taking 0-10V output as an example, when the output signal is 5v, the current wind speed is calculated. The span of wind speed range is 30m/s, which is expressed by 10V voltage signal. 30M/s/10V = 3M/s/V, that is, the change of wind speed is 3m/s for every 1V of voltage. The measured value is 5v-0v = 5v. 5V * 3/m/s/V = 15m/s. The current wind speed is 15 m/s.